Name: Junzhe Wu NetID: junzhew3

Q4:

- Sending eight bits of data per character with the required start and stop bits will take up 10 times the bit time for ach byte sent. At 9600 baud, the bit time is about 1/9600 microseconds which makes each character sent take 10/9600 milliseconds.

- X = 4

- X = 13

- X = 941

- X = 59

- X = 832

- X = 64

Q5:

char, int, float, double, void

Q6:

int: 32 bits

long int: 40 bits

unsigned int: 32 bits

short int: 16 bits

char: 8 bits

double: 64 bits

float: 32 bits

Q7:

int: 16 bits

long int: 32 bits

unsigned int: 16 bits

short int: 16 bits

char: 8 bits

double: 32 bits

float: 32 bits

Q8:

Q9:

#include "msp430g2553.h"

#include "UART.h"

void print_every(int rate);

char get_switchstate(void);

char newprint = 0;

long NumOn = 0;

long NumOff = 0;

int statevar = 1;

int timecheck = 0;

int x = 0;

int state = 0;

void main(void) {

WDTCTL = WDTPW + WDTHOLD;                 // Stop WDT

if (CALBC1_16MHZ ==0xFF || CALDCO_16MHZ == 0xFF) while(1);

DCOCTL = CALDCO_16MHZ;    // Set uC to run at approximately 16 Mhz

BCSCTL1 = CALBC1_16MHZ;

// Initialize Port 1

P1SEL &= ~0x01;

P1SEL2 &= ~0x01;

P2SEL &= ~0xc0;

P2SEL2 &= ~0xc0;

P1REN = 0x0;  // No resistors enabled for Port 1

P1DIR |= 0xf0; // Set P1,4,P1.5, P1.6, and P1.7 to output

P2DIR &= ~0xc0; //Set P2.6 and P2.7 to input

P2REN |= 0xc0;  // P2.6 and P2.7 Resistor enabled

P2OUT |= 0xc0;  // P2.6 and P2.7 Pullup Resistor selected

// Timer A Config

TACCTL0 = CCIE;       		// Enable Periodic interrupt

TACCR0 = 16000;                // period = 1ms

TACTL = TASSEL_2 + MC_1; // source SMCLK, up mode

Init_UART(115200,1);

_BIS_SR(GIE); 		// Enable global interrupt

/*

x =   495 & ( 0x5 << 4);

UART_printf("%d\n\r",x);

*/

while(1) {  // Low priority Slow computation items go inside this while loop.  Very few (if anyt) items in the HWs will go inside this while loop

// for use if you want to use a method of receiving a string of chars over the UART see USCI0RX_ISR below

//      if(newmsg) {

//          newmsg = 0;

//      }

// The newprint variable is set to 1 inside the function "print_every(rate)" at the given rate

if ( (newprint == 1) && (senddone == 1) )  { // senddone is set to 1 after UART transmission is complete

// only one UART_printf can be called every 15ms

//UART_printf("St%d On %ld Off %ld\n\r",statevar,NumOn,NumOff);

UART_printf("State: %d \n\r",statevar);

newprint = 0;

}

}

}

// Timer A0 interrupt service routine

//Q9

#pragma vector=TIMER0_A0_VECTOR

__interrupt void Timer_A (void)

{

timecheck++; // Keep track of time for main while loop.

print_every(500);

switch (statevar) {

case 1:  //LED1 ON

P1OUT = 0x10;

if (timecheck == 500) {

timecheck = 0;

statevar = 2;  // Next Timer_A call go to state 2

} else

break;

case 2:  //LED2 ON

P1OUT = 0x20;

if (timecheck == 500) {

timecheck = 0;

statevar = 3;

} else

break;

case 3:  //LED3 ON

P1OUT = 0x40;

if (timecheck == 500) {

timecheck = 0;

statevar = 4;

} else

break;

case 4:  //LED4 ON

P1OUT = 0x80;

if (timecheck == 500) {

timecheck = 0;

statevar = 5;

} else

break;

case 5:  //LED3 ON

P1OUT = 0x40;

if (timecheck == 500) {

timecheck = 0;

statevar = 6;

}

break;

case 6:  //LED2 ON

P1OUT = 0x20;

if (timecheck == 500) {

timecheck = 0;

statevar = 1;

} else

break;

}

}

/*

// ADC 10 ISR - Called when a sequence of conversions (A7-A0) have completed

#pragma vector=ADC10_VECTOR

__interrupt void ADC10_ISR(void) {

}

*/

// USCI Transmit ISR - Called when TXBUF is empty (ready to accept another character)

#pragma vector=USCIAB0TX_VECTOR

__interrupt void USCI0TX_ISR(void) {

if(IFG2&UCA0TXIFG) {		// USCI_A0 requested TX interrupt

if(printf_flag) {

if (currentindex == txcount) {

senddone = 1;

printf_flag = 0;

IFG2 &= ~UCA0TXIFG;

} else {

UCA0TXBUF = printbuff[currentindex];

currentindex++;

}

} else if(UART_flag) {

if(!donesending) {

UCA0TXBUF = txbuff[txindex];

if(txbuff[txindex] == 255) {

donesending = 1;

txindex = 0;

}

else txindex++;

}

} else {  // interrupt after sendchar call so just set senddone flag since only one char is sent

senddone = 1;

}

IFG2 &= ~UCA0TXIFG;

}

if(IFG2&UCB0TXIFG) {	// USCI_B0 requested TX interrupt (UCB0TXBUF is empty)

IFG2 &= ~UCB0TXIFG;   // clear IFG

}

}

// USCI Receive ISR - Called when shift register has been transferred to RXBUF

// Indicates completion of TX/RX operation

#pragma vector=USCIAB0RX_VECTOR

__interrupt void USCI0RX_ISR(void) {

if(IFG2&UCB0RXIFG) {  // USCI_B0 requested RX interrupt (UCB0RXBUF is full)

IFG2 &= ~UCB0RXIFG;   // clear IFG

}

if(IFG2&UCA0RXIFG) {  // USCI_A0 requested RX interrupt (UCA0RXBUF is full)

//    Uncomment this block of code if you would like to use this COM protocol that uses 253 as STARTCHAR and 255 as STOPCHAR

/*		if(!started) {	// Haven't started a message yet

if(UCA0RXBUF == 253) {

started = 1;

newmsg = 0;

}

}

else {	// In process of receiving a message

if((UCA0RXBUF != 255) && (msgindex < (MAX_NUM_FLOATS*5))) {

rxbuff[msgindex] = UCA0RXBUF;

msgindex++;

} else {	// Stop char received or too much data received

if(UCA0RXBUF == 255) {	// Message completed

newmsg = 1;

rxbuff[msgindex] = 255;	// "Null"-terminate the array

}

started = 0;

msgindex = 0;

}

}

*/

IFG2 &= ~UCA0RXIFG;

}

}

// This function takes care of all the timing for printing to UART

// Rate determined by how often the function is called in Timer ISR

int print_timecheck = 0;

void print_every(int rate) {

if (rate < 15) {

rate = 15;

}

if (rate > 10000) {

rate = 10000;

}

print_timecheck++;

if (print_timecheck == rate) {

print_timecheck = 0;

newprint = 1;

}

}

char get_switchstate(void){

char state1 = 1;

char P6 = 0;

char P7 = 0;

P6 = 0x40 & P2IN;

P7 = 0x80 & P2IN;

if ((P6 == 64) && (P7 == 128)){

state1 = 0;

}

if ((P6 == 0) && (P7 == 128)){

state1 = 1;

}

if ((P6 == 64) && (P7 == 0)){

state1 = 2;

}

if ((P6 == 0) && (P7 == 0)){

state1 = 3;

}

return state1;

}

Q10:

#include "msp430g2553.h"

#include "UART.h"

void print_every(int rate);

char get_switchstate(void);

char newprint = 0;

long NumOn = 0;

long NumOff = 0;

int statevar = 1;

int timecheck = 0;

int x = 0;

int state = 0;

void main(void) {

WDTCTL = WDTPW + WDTHOLD;                 // Stop WDT

if (CALBC1_16MHZ ==0xFF || CALDCO_16MHZ == 0xFF) while(1);

DCOCTL = CALDCO_16MHZ;    // Set uC to run at approximately 16 Mhz

BCSCTL1 = CALBC1_16MHZ;

// Initialize Port 1

P1SEL &= ~0x01;

P1SEL2 &= ~0x01;

P2SEL &= ~0xc0;

P2SEL2 &= ~0xc0;

P1REN = 0x0;  // No resistors enabled for Port 1

P1DIR |= 0xf0; // Set P1,4,P1.5, P1.6, and P1.7 to output

P2DIR &= ~0xc0; //Set P2.6 and P2.7 to input

P2REN |= 0xc0;  // P2.6 and P2.7 Resistor enabled

P2OUT |= 0xc0;  // P2.6 and P2.7 Pullup Resistor selected

// Timer A Config

TACCTL0 = CCIE;       		// Enable Periodic interrupt

TACCR0 = 16000;                // period = 1ms

TACTL = TASSEL_2 + MC_1; // source SMCLK, up mode

Init_UART(115200,1);

_BIS_SR(GIE); 		// Enable global interrupt

/*

x =   495 & ( 0x5 << 4);

UART_printf("%d\n\r",x);

*/

while(1) {  // Low priority Slow computation items go inside this while loop.  Very few (if anyt) items in the HWs will go inside this while loop

// for use if you want to use a method of receiving a string of chars over the UART see USCI0RX_ISR below

//      if(newmsg) {

//          newmsg = 0;

//      }

// The newprint variable is set to 1 inside the function "print_every(rate)" at the given rate

if ( (newprint == 1) && (senddone == 1) )  { // senddone is set to 1 after UART transmission is complete

// only one UART_printf can be called every 15ms

//UART_printf("St%d On %ld Off %ld\n\r",statevar,NumOn,NumOff);

UART_printf("State: %d \n\r",state);

newprint = 0;

}

}

}

//Q10

#pragma vector=TIMER0_A0_VECTOR

__interrupt void Timer_A (void)

{

timecheck++; // Keep track of time for main while loop.

print_every(500);  // units determined by the rate Timer_A ISR is called, print every "rate" calls to this function

state = get_switchstate();

switch (state) {

case 0:  //LED1 Blinking

if (timecheck == 1) {

P1OUT = 0x10;

}

else if (timecheck == 500) {

P1OUT = 0x00;

}

else if (timecheck == 1000) {

timecheck = 0;

}

break;

case 1:  //LED2 Blinking

if (timecheck == 1) {

P1OUT = 0x20;

}

else if (timecheck == 500) {

P1OUT = 0x00;

}

else if (timecheck == 1000) {

timecheck = 0;

}

break;

case 2:  //LED3 Blinking

if (timecheck == 1) {

P1OUT = 0x40;

}

else if (timecheck == 500) {

P1OUT = 0x00;

}

else if (timecheck == 1000) {

timecheck = 0;

}

break;

case 3:  //LED4 Blinking

if (timecheck == 1) {

P1OUT = 0x80;

}

else if (timecheck == 500) {

P1OUT = 0x00;

}

else if (timecheck == 1000) {

timecheck = 0;

}

break;

}

}

/*

// ADC 10 ISR - Called when a sequence of conversions (A7-A0) have completed

#pragma vector=ADC10_VECTOR

__interrupt void ADC10_ISR(void) {

}

*/

// USCI Transmit ISR - Called when TXBUF is empty (ready to accept another character)

#pragma vector=USCIAB0TX_VECTOR

__interrupt void USCI0TX_ISR(void) {

if(IFG2&UCA0TXIFG) {		// USCI_A0 requested TX interrupt

if(printf_flag) {

if (currentindex == txcount) {

senddone = 1;

printf_flag = 0;

IFG2 &= ~UCA0TXIFG;

} else {

UCA0TXBUF = printbuff[currentindex];

currentindex++;

}

} else if(UART_flag) {

if(!donesending) {

UCA0TXBUF = txbuff[txindex];

if(txbuff[txindex] == 255) {

donesending = 1;

txindex = 0;

}

else txindex++;

}

} else {  // interrupt after sendchar call so just set senddone flag since only one char is sent

senddone = 1;

}

IFG2 &= ~UCA0TXIFG;

}

if(IFG2&UCB0TXIFG) {	// USCI_B0 requested TX interrupt (UCB0TXBUF is empty)

IFG2 &= ~UCB0TXIFG;   // clear IFG

}

}

// USCI Receive ISR - Called when shift register has been transferred to RXBUF

// Indicates completion of TX/RX operation

#pragma vector=USCIAB0RX_VECTOR

__interrupt void USCI0RX_ISR(void) {

if(IFG2&UCB0RXIFG) {  // USCI_B0 requested RX interrupt (UCB0RXBUF is full)

IFG2 &= ~UCB0RXIFG;   // clear IFG

}

if(IFG2&UCA0RXIFG) {  // USCI_A0 requested RX interrupt (UCA0RXBUF is full)

//    Uncomment this block of code if you would like to use this COM protocol that uses 253 as STARTCHAR and 255 as STOPCHAR

/*		if(!started) {	// Haven't started a message yet

if(UCA0RXBUF == 253) {

started = 1;

newmsg = 0;

}

}

else {	// In process of receiving a message

if((UCA0RXBUF != 255) && (msgindex < (MAX_NUM_FLOATS*5))) {

rxbuff[msgindex] = UCA0RXBUF;

msgindex++;

} else {	// Stop char received or too much data received

if(UCA0RXBUF == 255) {	// Message completed

newmsg = 1;

rxbuff[msgindex] = 255;	// "Null"-terminate the array

}

started = 0;

msgindex = 0;

}

}

*/

IFG2 &= ~UCA0RXIFG;

}

}

// This function takes care of all the timing for printing to UART

// Rate determined by how often the function is called in Timer ISR

int print_timecheck = 0;

void print_every(int rate) {

if (rate < 15) {

rate = 15;

}

if (rate > 10000) {

rate = 10000;

}

print_timecheck++;

if (print_timecheck == rate) {

print_timecheck = 0;

newprint = 1;

}

}

char get_switchstate(void){

char state1 = 1;

char P6 = 0;

char P7 = 0;

P6 = 0x40 & P2IN;

P7 = 0x80 & P2IN;

if ((P6 == 64) && (P7 == 128)){

state1 = 0;

}

if ((P6 == 0) && (P7 == 128)){

state1 = 1;

}

if ((P6 == 64) && (P7 == 0)){

state1 = 2;

}

if ((P6 == 0) && (P7 == 0)){

state1 = 3;

}

return state1;

}

Q11:

Q12:

Q13: